Drive Test 工作心得（Benchmark 邮件 + 测试复盘）

VIP 9 Road Benchmark Drive Test  |  2026-08-29 ~ 31 受理，2026-09-01 复盘

# 一、工作概览与计划

本次任务是 VIP 9 Road 基准路测（Benchmark Drive Test），对比三家越南运营商在同一路线的网络性能，用于评估覆盖与速率表现。

- 范围：9 条测试路线（主城区往返路线）

- 运营商：Viettel、Mobifone、Vinaphone（三家对比）

- 测试项：FTP UL / FTP DL（LTE 4G 与 NR 5G）；MOS（VoLTE 语音质量）

- 采集指标：RSRP / SINR / PCI / 吞吐量

- 测试日期：2026-08-29 ~ 08-31；今天（09-01）安排 Route 8 retest、Route 9 数据整理

路线清单如下：

| No. | Route Description |
| --- | --- |
| 1 | from MBF Office to Government Office |
| 2 | from MBF Office to MPS Branch Office |
| 3 | from MBF Office to MPS HQ |
| 4 | from MPS Branch Office to MPS HQ |
| 5 | from MPS Branch Office to Government Office |
| 6 | MBF Office to Northern MBF |
| 7 | from MPS HQ to Government Office |
| 8 | from MPS HQ to Vinhomes Riverside |
| 9 | from MPS HQ to National Data Center |

（注：路线均为河内市区 / 近郊往返段，Route 9 属于最远的一条。）

# 二、测试执行结果

| Route | 网络/运营商 | FTP UL / FTP DL | 备注 |
| --- | --- | --- | --- |
| 1 ~ 7, 9 | LTE / NR (Viettel, Mobifone, Vinaphone) | FTP UL / FTP DL 完成 | MOS 未测（SIM 卡故障） |
| 8 | LTE / NR | FTP UL 完成；FTP DL 未完成 | Viettel 4G / VNPT 4G SIM 余额不足，需重测；MOS 未测 |

总体完成度：FTP UL/DL 大部分完成（LTE/NR × 三运营商），MOS 因 SIM 卡故障全部未测；Route 8 因 SIM 余额不足导致 FTP DL 未完成。

# 三、遇到的问题与处理过程

本次工作中最核心的问题是 SIM 卡相关的三个连锁问题，直接影响测试完成度：

| 问题 | 原因 | 处理 / 计划 |
| --- | --- | --- |
| MOS 全未测 | Viettel & VNPT 的 MOS SIM 卡故障（VoLTE MOS 无法采集） | 已通知 Ouyang；新卡到位后 9 条 route 补测 MOS |
| Route 8 FTP DL 未完成 | Viettel 4G / VNPT 4G SIM 卡余额不足，任务无法完成 | 申请新 SIM；新卡到位后重测（今天已安排 retest） |
| Route 9 耗时长 | 路线过远，完成耗时约 2.5 小时；司机还错过一个路口多跑了路线 | 评估拆分路段 / 精简测试项 |
| 全部手机 SIM 低余额 | 所有测试手机余额偏低，剩余任务无法可靠执行 | 急需带新余额的新 SIM 卡，持续跟进 Ouyang |

沟通与跟进记录：

- 已通知 Ouyang：MOS SIM 卡故障、Viettel 4G / VNPT 4G 余额不足、全部手机余额偏低，急需新卡

- 当前状态：新 SIM 卡暂无回音（no news so far），需持续跟进闭环

# 四、邮件写作心得（重点）

## 4.1 邮件六大板块固定结构

所有 RF / 路测邮件都按以下 6 块组织，顺序固定，保证可扫读性：

- ① Greeting + Overview —— 一句话说明邮件性质（Today's plan / result / update），必须加 “This is just a plan (tentative) and may change.”

- ② Change / Precondition note —— 变更原因、前置条件（如 SIM 卡无余额、测试改期）

- ③ Test plan table —— 主体内容，用 HTML 表格（No. | Route | Type | Test Case | Status | Remarks）

- ④ Reference images —— 路线图 / 站点图 / 截图，用 CID 内联图（cid:image001）

- ⑤ Vehicle / Personnel / Time —— 车辆、司机、开始/结束时间（如需要）

- ⑥ Important notice + Disclaimer + 固定签名

## 4.2 HTML 表格与状态标注

- 表格列建议：No. | Route Description | Operator | Type | FTP UL | FTP DL | MOS | Remarks

- 状态颜色规范：绿色 ✓ = 完成；红色 ✗ = 未测 / 未完成；⚠ Incomplete = 部分未完成（需在 Remarks 写原因）

- 扩展运营商维度：每个 Route 按 Viettel / Mobifone / Vinaphone 展开，每运营商再分 LTE / NR 两行

## 4.3 常用英文措辞模板

- 总体表述：‘Please find below the benchmark drive test result for VIP 9 Road conducted from 29 to 31 Aug 2026, covering 3 operators: Viettel, Mobifone and Vinaphone.’

- 计划提示：‘This is just a plan (tentative) and may change.’

- 重测原因：‘Route 8 FTP DL was attempted, but due to Viettel 4G & VNPT 4G SIM cards not having enough balance, the task cannot be completed as planned. New SIM cards have been requested (Ouyang informed); retest will be arranged once available.’

- MOS 未测原因：‘MOS is not tested – MOS SIM card malfunction for Viettel and VNPT.’

- 紧急诉求：‘We desperately need new SIM cards with new balance to continue the remaining benchmark tasks.’

- 时长说明：‘Route 9 is too far and took almost 2 and a half hours to finish.’

## 4.4 发送规则与注意事项

- 收件人：demo 邮件仅发送 jehovah.yii.hon1@h-partners.com（未经授权不得发给业务收件人）

- 主题格式：‘Today's Tasks (dd/MMM) - topic’、‘VIP 9 Road Benchmark Drive Test Result (29-31 Aug) - Viettel / Mobifone / Vinaphone’、‘Update - ...’

- HTML 正文必须经 convert（--convert-html）保证 Outlook 兼容；图片用 CID 内联（--inline-image ‘image001=路径’）

- 发送前 checklist：主题、收件人、表格数据、状态颜色、原因说明、固定签名

- 一次只表达一个核心诉求（如本次 = SIM 卡），表格给全量事实，正文给结论与行动项

# 五、经验教训（写给自己）

- 最大教训：测试前必须核查 SIM 卡状态——4G 流量余额 + MOS（VoLTE）卡是否正常，缺一不可；本次 3 个测试受阻全部源于 SIM 卡

- 依赖项要有闭环：外场资源（新 SIM 卡）不是通知一次就完成，需要书面邮件 + 跟进节点（deadline），本次 ‘no news so far’ 就是没有闭环的代价

- 路线规划要考虑时长：Route 9 耗时 2.5 小时过长，长路线应拆分或精简测试项，避免一整天压在一段路上

- 司机执线要有防错机制：错过路口会多跑路线并污染计划时间轴，出发前确认导航与路线图

- 未完成项必须留痕：Status 标红 + Remarks 写原因，保证 data traceability，补测时可直接溯源（本次邮件已体现了这个习惯）

- 邮件模板化提升效率：固定 6 块结构 + HTML 表格 + 措辞模板，让一封专业邮件在几分钟内完成，且收件方只需扫一眼就能拿到全部事实

# 六、后续行动项（TODO）

- 跟进 Ouyang：新 SIM 卡（Viettel 4G / VNPT 4G / MOS 卡）申请状态，持续到有确认

- 新卡到位后：Route 8 FTP DL 重测（今天已安排 retest）

- MOS 补测：9 条 route 全部补 VoLTE MOS（Viettel / Mobifone / Vinaphone）

- 与 Ouyang 讨论 Route 9 拆分 / 精简方案，控制单条路线时长

- 建立 SIM 卡检查清单：每轮测试出发前核查 4G 余额 + MOS 卡状态

To verify which site on air , find Nguyen Trong Giap 00079600

To know on air status, please check group [War Room] MBF 5G project - Ha Noi cutover

Thurday

- Afternoon: MBF customer meeting (You do not need to attend)

- Night: You need to report about benchmark Drive Test and report progress

Saturday

- TMO Meeting

## CQT — Call Quality Test

- What it is: A test methodology (a comprehensive test campaign)

- How it's performed: At fixed/stationary test points selected in advance (indoors and outdoors), the tester dials actual voice calls and evaluates multiple KPIs

- What it measures: A broad set of indicators, including:

  - Call Setup Success Rate (Accessibility)

  - Call Drop Rate (Retainability)

  - Coverage Rate

  - Receive Quality (RxQual)

  - Unidirectional Talk / Crosstalk Rate (Voice Quality)

- Purpose: Overall network performance validation at specific locations

## MOS — Mean Opinion Score

- What it is: A single quantitative KPI/metric that measures perceived voice quality

- How it's calculated:

  - Objective: via the PESQ (Perceptual Evaluation of Speech Quality) algorithm

  - Subjective: by human listeners rating quality on a 1–5 scale

  - Final MOS = average of all individual call scores: MOS = (M₁ + M₂ + ... + Mₙ) / n

- Score range: Typically 1.0 – 5.0 (≥3.0 is generally acceptable; ≥4.0 is good quality)

- Where it's measured: Can be tested during both DT (Drive Test) and CQT

## Relationship

| Aspect | CQT | MOS |
| --- | --- | --- |
| Nature | Test methodology | Quality metric |
| Scope | Multiple KPIs (accessibility, retainability, coverage, etc.) | One KPI (voice quality only) |
| MOS in CQT? | Yes — MOS is one of the sub-items evaluated during CQT | Measured as part of CQT or DT |
| Analogy | Like a "comprehensive exam" | Like the "grade" for one subject |

In short: CQT is the test you perform; MOS is one of the scores you get from that test (specifically for voice quality). During a CQT, you might also check call setup success, drop rates, coverage, and unidirectional talk — MOS is just the voice quality dimension among them.